CHAPTER 62-524

NEW POTABLE WATER WELL PERMITTING

IN DELINEATED AREAS

INDEX

PAGE

62-524.100	Intent Of New Potable Water Well Permitting In Delineated Areas.  (Repealed)	2

62-524.150	Scope Of New Potable Water Well Permitting In Delineated Areas.  (Repealed)	2

62-524.200	Definitions For New Potable Water Well Permitting In Delineated Areas.	2

62-524.300	General Requirements For New Potable Water Well Permitting In Delineated Areas.  (Repealed)	3

62-524.400	Delineation of Areas for Application of New Potable Water Well Permitting.  (Repealed)		3

62-524.410	Data For Delineation Of Areas For Application Of New Potable Water Well Permitting.  (Repealed)	3

62-524.420	Procedures For Delineation Of Areas For Application Of New Potable Water Well Permitting.	3

62-524.430	Maps Containing Delineated Areas.	6

62-524.500	Well Location Requirements for New Potable Water Well Permitting in Delineated Areas.  (Repealed)	11

62-524.550	Well Construction Requirements for New Potable Water Well Permitting in Delineated Areas.	11

62-524.600	Water Quality Testing for New Potable Water Well Permitting in Delineated Areas.		13

62-524.650	Clearing for Use of New Potable Water Wells in Delineated Areas.	13

62-524.700	Permit Requirements for New Potable Water Wells in Delineated Areas.	13

62-524.710	Exemption from New Potable Water Well Permitting in Delineated Areas.	14

62-524.720	Fees for New Potable Water Wells in Delineated Areas. (Repealed)	14

62-524.730	Inspections of New Potable Water Wells in Delineated Areas. (Repealed )	14

62-524.740	Violations and Penalties for New Potable Water Wells in Delineated Areas.	14

62-524.800	Delegation of New Potable Water Well Permitting, Testing and Clearance in Delineated Areas.  (Repealed)	15

62-524.900	Data Forms for New Potable Water Well Permitting in Delineated Areas.  (Repealed)		15

62-524.910	Data Reporting for New Potable Water Well Permitting in Delineated Areas.  (Repealed)		15

# 62-524.100	Intent Of New Potable Water Well Permitting In Delineated Areas.  (Repealed)

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309, 376.307 FS. History - New 5-16-89, Formerly 17-524.100, Repealed 12-9-96.

# 62-524.150	Scope Of New Potable Water Well Permitting In Delineated Areas.  (Repealed)

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309, 376.307 FS. History - New 5-16-89, Formerly 17-524.150, Repealed 12-9-96.

# 62-524.200	Definitions For New Potable Water Well Permitting In Delineated Areas.

(1)	"Available Potable Water System" means, for the purpose of this Chapter, a public water system, as defined in Rule 62-550.200, F.A.C., which has sufficient capacity and is legally able to serve specific additional connections.

(2)	"Delineated Area" means a surface area identified pursuant to Rule 62-524.420, F.A.C., within which ground water contamination is known to exist or which encompasses vulnerable areas or areas in which the Department provides a subsidy for restoration or replacement of contaminated drinking water supplies.

(3)	"Ground Water Contamination" means, for the purpose of this Chapter, the presence outside an applicable zone of discharge in Class F-I, G-I, or G-II ground water of one or more substances in quantities which exceed a primary drinking water maximum contaminant level as set forth in Chapter 62-550, F.A.C., present an imminent hazard pursuant to Section 403.855, F.S., or for which the State Health Officer in the Department of Health based upon a written request from the Department, has advised the Department in writing is present in deleterious amounts.  The determination, under this Section, of the existence of ground water contamination based upon the presence of deleterious amounts shall not constitute the establishment of a standard under either Chapter 62-520 or Chapter 62-550, F.A.C.  If the concentration of any primary drinking water standard in the natural background quality of the ground water is greater than the stated maximum contaminant level, the representative background value shall be the prevailing standard.

(4)	"New Potable Water Well" means any excavation that is drilled or bored, or converted from non-potable water use, after delineation in an area delineated pursuant to Rule 62-524.400, F.A.C., when the intended use of such excavation is for the location and acquisition of ground water which supplies water for human consumption. This does not include repair of an existing potable water well.

(5)	"Vulnerable area" is an area in which research or monitoring data indicate that ground water is vulnerable to nitrate contamination because of the presence of potential sources of nitrate contamination, and because of land surface and subsurface characteristics.

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309, 376.307 FS. History - New 5-16-89, Amended 3-3-92, Formerly 17-524.200, Amended 2-7-95.

# 62-524.300	General Requirements For New Potable Water Well Permitting In Delineated Areas.  (Repealed)

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309 FS. History - New 5-16-89, Amended 3-3-92, Formerly 17-524.300, Amended 2-7-95, Repealed 12-9-96.

# 62-524.400	Delineation of Areas for Application of New Potable Water Well Permitting.  (Repealed)

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309, 376.307 FS. History - New: 5-16-89, Formerly 17-524.400, Amended 2-7-95, Repealed 12-9-96.

# 62-524.410	Data For Delineation Of Areas For Application Of New Potable Water Well Permitting.  (Repealed)

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309, 376.307 FS. History - New: 5-16-89, Formerly 17-524.410, Repealed 12-9-96.

# 62-524.420	Procedures For Delineation Of Areas For Application Of New Potable Water Well Permitting.

(1)	Based upon available data, the Department shall identify and locate, for the purpose of application of the requirements of this Chapter, areas within which ground water contamination is known to exist or which encompasses vulnerable areas or areas in which the Department provides a subsidy for restoration or replacement of contaminated drinking water supplies.

(2)	The Department shall rely on data from samples collected and analyzed using Department approved quality assurance/quality control procedures. Where quality assurance/quality control procedures are not documented the Department shall evaluate the data for completeness and accuracy in order to determine acceptability for use in delineation under this Chapter.

(3)	Sources of ground water data to be used for delineation of areas under this Chapter shall include:

(a)	Local, state, and federal agencies.

(b)	Water management districts.

(c)	Department programs.

(4)	For wells, sites, or sources with known ground water contamination, where insufficient site specific ground water data exist for determination of contaminant plume boundaries, a delineated area shall be established in the following manner:

(a)	A 1000-foot setback from the well, site or source boundary.

(b)	Where data on the distribution or movement of ground water contamination indicate that a 1000-foot setback is insufficient the Department shall establish an alternate setback based on such data.

(5)	For sites with a history of application of ethylene dibromide where insufficient site specific ground water data exist for determination of contaminant plume boundaries, the Department shall delineate an area which encompasses the area of application and a setback, based on data on the distribution of ethylene dibromide contamination, or a 1000-foot setback, whichever is larger.

(6)	For sites where a hydrogeologic investigation of ground water has been conducted and the nature and extent of a contaminant plume is documented and sufficient data exist for predictive ground water modelling, the Department shall delineate an area which encompasses the ground water contamination and its predicted movement for the next two years.

(7)	Where the source or site which resulted in an area being delineated is the subject of remediation for ground water clean-up, the effect of this remediation shall be considered by the Department in subsequent delineation updates.

(8)	For areas in which the Department provides a subsidy for restoration or replacement of contaminated drinking water supplies through extending existing water lines or developing new water supply systems under section 376.307(4)(b)3 and (c), F.S., the Department shall delineate an area which encompasses such extended water lines or water lines constructed as part of a new water system and a 1000-foot setback.

(9)	For areas in which the Department determines that ground water is vulnerable to contamination with nitrate, the Department shall delineate such vulnerable areas.  The Department shall determine where vulnerable areas exist by using the following information when available:

(a)	Physical properties of soils

(b)	Vadose zone media

(c)	Hydrogeologic characteristics of aquifer systems

(d)	Depth to ground water

(e)	Recharge

(f)	Karst features

(g)	Topography

(h)	Presence of Class G-II ground water or other potable ground water with less than 10,000 mg/L total dissolved solids

(i)	Water quality data; and

(j)	Nitrogen application or loading rates for potential sources of nitrate contamination.

(10)	In delineating areas under this rule, the Department shall coordinate with other affected agencies, particularly those receiving delegation under Rule 62-524.800, F.A.C., in the technical aspects of delineation.

(11)	The Department shall present delineated areas to the Environmental Regulation Commission for approval at rulemaking public hearings duly noticed as required by Section 120.54, F.S.

(a)	At such public hearings the Commission, when approving delineated areas, shall consider the known ground water contamination and its projected movement until the next delineation update.

(b)	If requested by the Commission, the Department shall present the data, predictive ground water modelling, and mapping procedure used to delineate each area presented to the Commission.

(c)	The Commission shall consider any other competent evidence regarding delineated areas.

(d)	Approval by the Commission of a delineated area shall result in that area being included on maps or other means of location and description prepared by the Department as described in subsections (12) and (13).  Each approved map or other means of location and description shall contain an effective date and shall be made available as provided in subsections (12) and (13).

(12)	To facilitate the permitting process, the Department shall provide maps which indicate all sections which contain any portion of a delineated area.  Prior to construction of a new potable water well within a mapped section, the potential applicant should contact the appropriate permitting authority which shall determine if the proposed well is within a delineated area.  Such maps or other information shall be made available by the Department to interested persons upon written request and upon payment of appropriate costs.

(13)	Following each update, the Department shall make available to water management districts, regional planning councils, the Department of Health and county building and zoning departments, maps or other information on areas for application of the requirements of this Chapter.

(a)	Where maps are provided, they shall be of an appropriate scale as determined by the Department based on the accuracy and precision of the data.

(b)	For each delineated area the Department shall provide a list of those contaminants to be tested pursuant to Rule 62-524.600, F.A.C., and shall specify any casing or solvent bond restrictions.

(14)	Maps or other information on areas for application of the requirements of this Chapter shall be periodically updated by the Department. Additional areas, or revision to existing areas, for application of the requirements of this Chapter may be delineated at any time as technical information becomes available.

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309, 376.307 FS. History - New 5-16-89, Amended 3-25-90, 7-4-91, 5-6-93, Formerly 17-524.420, Amended 2-7-95, Amended 12-9-96.

# 62-524.430	Maps Containing Delineated Areas.

The following maps, which are incorporated herein by reference, show surface areas, delineated pursuant to Rule 62-524.420, F.A.C.  Each map listed contains a month and year which corresponds to the date the Department prepared the most recent map showing any portion of a delineated area.  Copies of these maps may be examined at the Department of Environmental Protection Bureau of Information Systems, or copies may be obtained, upon receipt of reproduction and other appropriate costs, from the Department of Environmental Protection Bureau of Information Systems, 2600 Blair Stone Road, Tallahassee, Florida 32399-2400.

(1)	ALACHUA COUNTY:

Archer 		11/94

Gainesville East	11/94

High Springs		11/94

High Springs SW	11/94

Micanopy		11/94

Monteocha		11/94

Newberry		11/94

Orange Heights	11/94

Waters Lake		11/94

(2)	BREVARD COUNTY:

Melbourne East	11/94

(3)	BROWARD COUNTY:

Cooper City		11/94

Fort Lauderdale North	11/94

Fort Lauderdale South	11/94

North Miami		11/94

Port Everglades	11/94

(4)	CITRUS:

Crystal River		11/94

Homosassa		11/94

(5)	COLUMBIA:

Columbia		11/94

Fort White		11/94

Lake City West	11/94

Mikesville		11/94

(6)	DADE COUNTY:

Hialeah		11/94

North Miami		11/94

South Miami		11/94

(7)	DESOTO:

Arcadia		11/94

(8)	DUVAL COUNTY:

Baldwin		11/94

Jacksonville		11/94

Jacksonville Heights	11/94

Marietta		11/94

(9)	ESCAMBIA COUNTY:

Cantonment		11/94

Pensacola		11/94

Seminole (AL)	11/94

West Pensacola	11/94

(10)	GILCHRIST:

High Springs SW	11/94

Waters Lake		11/94

(11)	GLADES COUNTY:

Moore Haven	11/94

(12)	HAMILTON:

Ellaville		11/94

Fort Union		11/94

(13)	HARDEE:

Griffins Corner	11/94

(14)	HERNANDO:

Masaryktown		11/94

Port Richey NE	11/94

Weekiwachee Springs	11/94

(15)	HIGHLANDS COUNTY:

Avon Park		11/94

Childs			11/94

Crewsville		11/94

Frostproof		11/94

Lake Arbuckle	11/94

Lake Arbuckle SW	11/94

Lake June In Winter	11/94

Lake Placid		11/94

Sebring		11/94

Venus SW		11/94

(16)	HILLSBOROUGH COUNTY:

Brandon		11/94

Citrus Park		11/94

Dover			11/94

Ft. Lonesome	11/94

Lithia			11/94

Lutz			11/94

Plant City West	11/94

Sulphur Springs	11/94

Tampa		11/94

Thonotosassa	11/94

Wimauma		11/94

(17)	INDIAN RIVER:

Vero Beach		11/94

(18)	JACKSON COUNTY:

Alford			11/94

Bascom		11/94

Campbellton		11/94

Cottondale East	11/94

Cottondale West	11/94

Cypress		11/94

Dellwood		11/94

Fairchild (GA)	11/94

Graceville 		11/94

Kynesville 		11/94

Malone		11/94

Marianna		11/94

Oakdale		11/94

Grangeburg (AL)	11/94

Saffold (AL)		11/94

Sills			11/94

Sneads		11/94

Steam Mill (GA)	11/94

(19)	LAKE COUNTY:

Astatula		11/94

Center Hill		11/94

Clermont East	11/94

Clermont West	11/94

Eustis			11/94

Howey In The Hills	11/94

Lake Louisa		11/94

Lake Louisa SW	11/94

Lake Nellie		11/94

Leesburg East	11/94

Mascotte		11/94

Sorrento		11/94

Umatilla		11/94

(20)	LEON COUNTY:

Tallahassee		11/94

(21)	LEVY:

Morriston		11/94

(22)	MADISON:

Cherry Lake		11/94

Madison		11/94

Nankin (GA)		11/94

Pinetta		11/94

(23)	MANATEE:

Ft. Lonesome	11/94

Wimauma		11/94

(24)	MARION COUNTY:

Belleview		11/94

Lady Lake		11/94

Lake Weir		11/94

Ocala East		11/94

Ocala West		11/94

Oxford		11/94

(25)	MARTIN COUNTY:

Indiantown		11/94

Okeechobee 4 SE	11/94

(26)	ORANGE COUNTY:

Apopka		11/94

Astatula		11/94

Clermont East	11/94

Eustis			11/94

Forest City		11/94

Lake Jessamine	11/94

Lake Louisa		11/94

Orlando East		11/94

Orlando West	11/94

Sorrento		11/94

Windermere		11/94

Winter Garden	11/94

(27)	OSCEOLA:

Ashton		11/94

Intercession City	11/94

Lake Louisa SW	11/94

Narcoossee		11/94

(28)	PASCO:

Lutz			11/94

(29)	PINELLAS:

Elfers			11/94

(30)	POLK COUNTY:

Alturas		11/94

Auburndale		11/94

Babson Park		11/94

Bartow		11/94

Bereah		11/94

Davenport		11/94

Dundee		11/94

Eloise			11/94

Frostproof		11/94

Gum Lake		11/94

Hesperides		11/94

Homeland		11/94

Intercession City	11/94

Lake Arbuckle	11/94

Lake Louisa SW	11/94

Lake Wales		11/94

Lake Weohyakapka	11/94

Lakeland		11/94

Mulberry		11/94

Nichols		11/94

Plant City East	11/94

Polk City		11/94

Providence		11/94

Socrum		11/94

Winter Haven	11/94

(31)	PUTNAM:

Baywood		11/94

(32)	SANTA ROSA:

Milton South		11/94

Pace			11/94

(33)	SEMINOLE COUNTY:

Aurantia		11/94

Bithlo			11/94

Casselberry		11/94

Forest City		11/94

Geneva		11/94

Sanford		11/94

Titusville SW		11/94

(34)	ST.JOHNS COUNTY:

Picolata		5/00

(35)	ST. LUCIE:

Fort Pierce NW	11/94

Okeechobee 1 NE	11/94

(36)	SUMTER:

Bushnell		11/94

Webster		11/94

(37)	SUWANNEE:

Dowling Park		11/94

Fort Union		11/94

Hildreth		11/94

Hillcoat		11/94

Live Oak East	11/94

O'Brien		11/94

(38)	VOLUSIA COUNTY:

Aurantia		11/94

De Land		11/94

Geneva		11/94

Orange City		11/94

Titusville SW		11/94

Rulemaking Authority 373.309, 403.061 FS. Law Implemented 373.309 FS. History - New 3-25-90, 10-4-90, 7-4-91, Formerly 17-524.430, Amended 2-7-95, 6-27-00.

# 62-524.500	Well Location Requirements for New Potable Water Well Permitting in Delineated Areas.  (Repealed)

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309 FS. History - New 5-16-89, Formerly 17-524.500, Repealed 12-9-96.

# 62-524.550	Well Construction Requirements for New Potable Water Well Permitting in Delineated Areas.

(1)	New potable water wells shall comply with the minimum construction standards contained in Rule 62-532.500, F.A.C.  Additional requirements may be assigned by the permitting authority relative to depth restrictions, location of screened or open hole interval, and length of casing where warranted by local specific information.

(2)	Methods for constructing new potable water wells shall be limited to rotary drilling, boring, or other method specifically approved by the permitting authority pursuant to Rule 62-524.700(1), F.A.C., which meets the water well construction criteria in Rule 62-532.500, F.A.C., except as required below.

(a)	Well casing and liner pipe shall be new, free of breaks, corrosion and dents, straight and true, and not out of round.  Welded or seamless black or galvanized steel pipe or casing, or stainless steel pipe or casing, or approved types of nonmetallic pipe shall be used for well casing or liner pipe.

(b)	Solvent-bonded couplings shall be prohibited in areas with known ground water contamination which includes solvent components.

(c)	To prevent the interchange of water and loss of artesian pressure.  Contaminated, unconfined ground water intervals shall be sealed off prior to drilling through the underlying confining interval.  Uncontaminated, unconfined ground water intervals shall be sealed off or otherwise protected prior to drilling into deeper, contaminated ground waters.

(d)	For any well casing installed in a bore hole, the annular space shall be filled from bottom to top with not less than a nominal two inch thickness of neat cement grout.

(e)	A concrete pad measuring three feet by three feet by four inches shall be constructed around the elevated portion of the casing so that the casing is centered in the pad to prevent soil erosion and seepage of surface contamination into the annular space.

(f)	A minimum elevation of one foot of casing above land surface shall be required.

(g)	A raw water tap shall be provided to allow sampling of the well before exposure to storage or treatment.

(h)	The well casing shall be visibly and permanently marked above the land surface with the latitude and longitude and the permit number issued by the permitting authority for that well.

(i)	To the extent practical, potable water wells shall be located outside an area delineated under Rule 62-524.420, F.A.C.

(j)	Where the source of contamination and the direction of ground water flow are known, in an area delineated under Rule 62-524.420, F.A.C., to the extent practical, potable water wells shall be located upgradient of the source.

(k)	New potable water wells shall be located on ground least subject to inundation.

(l)	Any new potable water well constructed within a delineated area that does not meet the construction standards of this section shall be abandoned and plugged in accordance with Rule 62-532.500, F.A.C., and applicable water management district rules.

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309 FS. History - New 5-16-89, Amended 3-25-90, Amended 3-3-92, Formerly 17-524.550, Amended 12-9-96.

# 62-524.600	Water Quality Testing for New Potable Water Well Permitting in Delineated Areas.

(1)	New potable water wells shall be tested using methods as specified in Rule 62-524.420, F.A.C., for the presence in the untreated water of the ground water contamination which resulted in the delineation.

(2)	The Department shall accept only test results obtained from water samples collected and analyzed by the Department of Health.  The well construction permit applicant shall be responsible for the cost of sample collection, shipping, and analysis.

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309 FS. History - New 5-16-89, Amended 3-3-92, 5-6-93, Formerly 17-524.600.

# 62-524.650	Clearing for Use of New Potable Water Wells in Delineated Areas.

(1)	If no ground water contamination is found upon testing of a new potable water well in a delineated area pursuant to Rule 62-524.600, F.A.C., the Department of Health shall be responsible for issuance of a letter of clearance to the well construction permit applicant.

(2)	If ground water contamination is found upon testing pursuant to Rule 62-524.600, F.A.C., or other ground water contamination is found, a well shall not be cleared for use without a demonstration, through water quality testing, that a filter or other permanent remedy prevents the users of the well from being exposed through ingestion, inhalation, or dermal absorption, as appropriate for a contaminant, to ground water contamination.

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309 FS. History - New 3-3-92, Formerly 17-524.650, Amended 12-9-96.

# 62-524.700	Permit Requirements for New Potable Water Wells in Delineated Areas.

(1)	A construction permit shall be obtained from the appropriate water management district pursuant to Rule 62-524.800, F.A.C., for all new potable water wells prior to installation or conversion.  Applicants shall submit a proposed well design with the completed application, and the permit fee, to the permitting authority.  Permit application shall be made under existing well construction permitting programs pursuant to Chapter 62-532, F.A.C., using forms adopted by the permitting authority for this purpose.  In addition to the general requirements of this Chapter, the permit shall address the following requirements through special conditions:

(a)	Well construction including method of construction, depth, location of cased and screened intervals, casing material and grouting.

(b)	Any special cleaning requirements for casing or drilling equipment.

(c)	Water quality testing.

(d)	Unique well identifiers where needed.

(2)	Permitting and construction of new potable water wells, except for a well to be used for a public water system as defined in Rule 62-550.200, F.A.C., are prohibited in delineated areas where a distribution line of an available potable water system is within 500 feet of the boundary of the property for which a well construction permit is being sought. Such prohibition applies unless the property owner or applicant obtains documentation from the public water system or the Department's Water Supply Restoration and Replacement Program, and submits such documentation to the permitting entity, which demonstrates either of the following:

(a)	That economic factors caused by physical or legal impediments to construction to a distribution line prevent the property owner or permit applicant from obtaining potable water through connection to the distribution line; or

(b)	That necessary water distribution line extensions (excluding plumbing and meters) cannot be completed within 30 days of application to the Department for water supply restoration or replacement.

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309 FS. History - New 5-16-89, Amended 3-3-92, Formerly 17-524.700, Amended 12-9-96.

# 62-524.710	Exemption from New Potable Water Well Permitting in Delineated Areas.

Exemption from the requirements of Rule 62-524.700, F.A.C., shall be granted to an applicant by the Department or the permitting authority upon demonstration using hydrogeological, water quality, and other pertinent information that the exemption will not result in the impairment of the intent and purpose of this Chapter.  Detailed requirements for each exemption shall be negotiated between the permit applicant and the permitting authority on a case by case basis.

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309, 373.326 FS. History - New 5-16-89, Formerly 17-524.710.

# 62-524.720	Fees for New Potable Water Wells in Delineated Areas. (Repealed)

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309 FS. History - New 5-16-89, Amended 3-3-92, Formerly 17-524.720, Repealed 2-16-12.

# 62-524.730	Inspections of New Potable Water Wells in Delineated Areas. (Repealed )

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309, 373.319 FS. History - New 5-16-89, Formerly 17-524.730, Repealed 2-16-12.

# 62-524.740	Violations and Penalties for New Potable Water Wells in Delineated Areas.

(1)	Prohibited Acts.

(a)	It shall be a violation of Section 373.309, F.S., and this Chapter to construct, convert from non-potable use, or abandon any potable water well, or use for human consumption any well subject to permit under this Chapter without having obtained a permit pursuant to Rule 62-524.700, F.A.C.  This prohibition shall apply to both the water well contractor and the well owner.

(b)	It shall be a violation of Section 373.309, F.S., and this Chapter to use for human consumption, after delineation, any water well subject to permit under this Chapter without having performed water quality testing pursuant to Rule 62-524.600, F.A.C.

(c)	It shall be a violation of Section 373.309, F.S., and this Chapter to use for human consumption, after delineation, any water well subject to permit under this Chapter in which contaminants have been found without a demonstration through water quality testing that a filter or other means of preventing the users of such a well from being exposed to ground water contamination is effective.

(2)	Penalties.

(a)	Any person who violates any provision of this Chapter, order, or permit issued under the authority of this Chapter shall, upon conviction, be guilty of a misdemeanor of the second degree, punishable as provided in Sections 775.082 and 775.083, F.S.  Continuing violation after an order or conviction shall constitute a separate violation for each day the violation occurs.

(b)	Any water well contractor who is in violation of paragraph (1)(a) shall, in addition to paragraph (2)(a), also be subject to the penalty provisions in Chapter 62-531, F.A.C, including the license suspension and revocation provisions contained therein.

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309, 373.323, 373.336 FS. History - New 5-16-89, Amended 3-25-90, Formerly 17-524.740.

# 62-524.800	Delegation of New Potable Water Well Permitting, Testing and Clearance in Delineated Areas.  (Repealed)

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 373.309 FS. History - New 5-16-89, Amended 3-3-92, Formerly 17-524.800, Repealed 12-9-96.

# 62-524.900	Data Forms for New Potable Water Well Permitting in Delineated Areas.  (Repealed)

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 120.55, 373.309 FS. History - New 5-16-89, Amended 3-25-90, Formerly 17-524.900, Repealed 12-9-96.

# 62-524.910	Data Reporting for New Potable Water Well Permitting in Delineated Areas.  (Repealed)

Rulemaking Authority 373.309, 403.061, 403.062 FS. Law Implemented 120.55, 373.309 FS. History - New 3-25-90, Formerly 17-524.910, Repealed 12-9-96.